VZ200/300 Quick Reference Card

By Leslie Milburn

Memory Map

| Address |  | Description |
| --- | --- | --- |
| Dec | Hex |  |
| 0 | 0 | BASIC Rom (Level II Standard) |
| 8192 | 2000 | BASIC Rom (VZ Extensions) |
| 16384 | 4000 | Reserved for Cartridges (DOS, Word Pro, Terminal, RTTY) |
| 26624 | 6800 | Memory Mapped I/O (Keyboard, Cassette, Screen, Speaker) |
| 28672 | 7000 | Video Display Ram |
| 30720 | 7800 | Communications Region (See Page 2) |
| 31465 | 7AE9 | Start of User Memory |
| 36864 | 9000 | End of 6K System (VZ200) |
| 44874 | AF4A | End of 16K System with DOS (VZ300) |
| 45184 | B800 | End of 16K System, No DOS (VZ300) |
| 53038 | CF2E | End of 24K System with DOS (VZ200 + 16K) |
| 53348 | D000 | End of 24K System, No DOS (VZ200 + 16K) |
| 63138 | F6A2 | End of 32K System with DOS (VZ300 + 16K) |
| 63488 | F800 | End of 32K System, No DOS (VZ300 + 16K) |
| 65525 | FEC9 | End of 64K System with DOS (VZ + 64K) |
| 65535 | FFFF | End of 64K System, No DOS (VZ + 64K) |

Port & Memory Mapped External Devices

| Port (Hex) | Memory Addr (Hex) | Description |
| --- | --- | --- |
| 00 - 0F | - | Printer Interface (Input/Output) |
| 01 | - | 8 Bit General Port (Input Only) |
| 10 - 1F | - | Disk Controller (Input/Output) |
| 20 - 2F | - 7000 - 77FF | Joystick (Input Only) Super Graphics (Output Only) |
| 30 - 3F | ? | Communications |
| 40 - 4F | ? | Light Pen (Input Only) |
| 50 - 5F | - | Real Time Clock (Proposed) |
| 60 - 6F | - | External Keyboard (Input/Output) |
| 70 - 7F | C000 - FFFF | Main Memory Bank Switch (Output Only) |
| C9 - CA | ? | Eprom Programmer |
| D8 - E7 | 8000 - 9FFF | Auto Boot (Proposed) |
| E8 - F7 | 6000 - 67FF | RDOS (Proposed) |
| F0 - F3 | ? | 24 Bit I/O Interface |
| F8 - FF | 4000 - 5FFF | Ram Disk (Proposed) |
| E0 - FF | ? | D. Newcombes Rom Board |
| FE | ? | 4K-64K Ram/Eprom |

- 1 -

Communications Region

| Address |  | Description |
| --- | --- | --- |
| Dec | Hex |  |
| 30720 | 7800 | RST Instruction Vectors |
| 30741 | 7815 | Keyboard Device Control Block |
| 30749 | 781D | Video Device Control Block |
| 30757 | 7825 | Printer Device Control Block |
| 30774 | 7836 | Keyboard Work Area |
| 30779 | 783B | Copy of Output Latch/Video Background (32 = Green, 48 = Buff) |
| 30845 - 7 | 787D - F | Interrupt Exit |
| 30862 - 3 | 788E - F | USR Subroutine Address |
| 30864 - 6 | 7890 - 2 | RND Number Seed |
| 30874 | 789A | Error Code Storage. (See Page 4) |
| 30876 | 789C | Output Device (0 = Video, 1 = Printer, 255 = Cassette) |
| 30880 - 1 | 78A0 - 1 | BASIC Stack Address/ Lower Boundary of String Space |
| 30882 - 3 | 78A2 - A3 | Current BASIC Line No/ Address of line with Error |
| 30884 - 5 | 78A4 - 5 | Start of Program Pointer |
| 30886 | 78A6 | Cursor Tab Position |
| 30895 | 78AF | Variable Type in WRA1 (2 = Integer, 3 = String, 4 = Sng Precison, 8 = Dbl Precision) |
| 30897 - 8 | 78B1 - 2 | Top of Memory Pointer |
| 30938 - 9 | 78DA - B | Line No of Last Data statement read |
| 30954 - 5 | 78EA - B | Line No in which error occurred. |
| 30969 - 70 | 78F9 - A | End of BASIC Program Pointer/ Start of SimpleVariables Table Pointer |
| 30971 - 2 | 78FB - C | Start of DIM Variables Table Pointer |
| 30973 - 4 | 78FD - E | Start of Free Memory Pointer |
| 30975 - 6 | 78FF - 900 | Pointer to Data Item Read |
| 30977-1002 | 7901 - 1A | Variable Declaration Table (26 Entries) |
| 31003 | 791B | Trace Flag (0 = Off, 1 = On) |
| 31004 - 57 | 791C - 51 | Reserved for Numeric Routines. e.g. Working Register Areas (WRA1 & 2) |
| 31058 - 204 | 7952 - E4 | Level II BASIC and DOS Exits. (Unused on standard VZ200/300) |

DOS Reserved Memory Area (Top 310 Bytes)

| Offset |  | Description |
| --- | --- | --- |
| Dec | Hex |  |
| 0 - 66 | 00 - 39 | Vectors (See Page3) |
| 67 - 76 | 43 - 4C | Sector Header (GAP2) |
| 77 - 204 | 4D - CC | Sector Data Buffer |
| 205 - 206 | CD - CE | Sector Checksum Storage |
| 207 - 230 | CF - E6 | Unused (?) |
| 231 - 310 | E7 - 136 | Track Map Buffer |

- 2 -

DOS Vector Description (pointed to by IY Regs)

| Offset |  | Description |
| --- | --- | --- |
| Dec | Hex |  |
| 0 | 0 | File Control Block No (0 = FCB1, 1 = FCB2) |
| 1 - 8 | 1 | Filename |
| 9 | 9 | Required File Type/Found File Type |
| 11 | 0B | Selected Drive Pattern (00H = Drive1, 80H = Drive2) |
| 12 | 0C | Open Requested Mode (0 = Read, 1 = Write) |
| 13 | 0D | Source Drive for Dcopy |
| 14 - 15 | 0E - 0F | User Buffer Address |
| 16 | 10 | Destination Drive for Dcopy |
| 17 | 11 | User Specified Sector Number |
| 18 | 12 | User Specified Track Number |
| 19 | 13 | Retry Count |
| 20 | 14 | Current Track Number |
| 21 | 15 | Next Sector Number |
| 22 | 16 | Next Track Number |
| 23 - 35 | 17 - 23 | File Control Block 1 (FCB1) |
| 36 - 48 | 24 - 30 | File Control Block 2 (FCB2) |
| 49 - 50 | 31 - 32 | Pointer to Data Buffer |
| 51 | 33 | Copy of I/O Latch |
| 52 - 53 | 34 - 35 | Pointer to Track/Sector Map |
| 54 | 36 | Track Count for Dcopy |
| 55 | 37 | Track Pointer for Dcopy |
| 56 | 38 | Stepper Phase |
| 57 - 66 | 39 - 42 | Reserved for Future Expansion |

File Control Block Description

| Offset |  | Description |
| --- | --- | --- |
| Dec | Hex |  |
| 0 | 00 | In Use Flag (0 = Not in Use, 1 = In Use) |
| 1 | 01 | Open Status (0 = Read, 1 = Write) |
| 2 - 10 | 02 - 0A | Filename |
| 11 | 0B | Track Number |
| 12 | 0C | Sector Number |
| 13 | 0D | Entry in Sector |

Common File Types

| T | Basic Editor File |  |
| --- | --- | --- |
| B | Binary File |  |
| D | Sequential Access Program Data File |  |
| F | Quickwrite Document |  |
| A | Russell Harrison's Edit Ass. File |  |
| S | Dave Mitchell/Mark Hardwood Edit Ass. File | Start Addr A280H |
| S | Quickwrite/Diskops System File/Label | Except Above |
| W | Edit Ass. with Diskops File | Start Addr A813H |
| W | E & F Word Pro with Patch 3.3 File | End Addr D000H |
| W | Russell Harrison Word Pro File | Except above two |

- 3 -

Standard Dos Internal Error Codes

| Error Code | Description |
| --- | --- |
| 0 | No Error |
| 1 | Syntax Error |
| 2 | File Already Exists |
| 3 | Directory Full |
| 4 | Disk Write Protected |
| 5 | File Not Open |
| 6 | Disk I/O Error |
| 7 | Disk Full |
| 8 | File Already Open |
| 9 | Sector Not Found |
| 10 | Checksum Error |
| 11 | Unsupported Device |
| 12 | File Type Mismatch |
| 13 | File Not Found |
| 14 | Disk Buffer Full |
| 15 | Illegal Read |
| 16 | Illegal Write |
| 17 | Break |

Basic Facts

| Item | Range Limits |
| --- | --- |
| Integer Variable | -32768 to 32767 |
| Single Precision Variable | -1.701411E±38 to 1.701411E±38 |
| Double Precision | -1.701411834544556D±38 to 1.701411834544556D±38 |
| Character String | 0 to 255 bytes |
| Line Number | 0 to 65529 |
| Coded Line Length | 0 to 64 bytes |

| Item | Minimum Memory Required (Storage) |
| --- | --- |
| Source Line | 5 bytes |
| Reserved Word | 1 byte |
| Operator | 1 byte |
| Variable Name | 2 bytes |
| Special Character | 1 byte |
| Constant Character | 1 byte |

| Item | Minimum Memory Required (Execution) |
| --- | --- |
| Integer Variable | 5 bytes |
| Single Precision Variable | 7 bytes |
| Double Precision Variable | 11 bytes |
| String Variable | 6 bytes |
| Array Variable | 12 bytes |
| For-Next Loop | 16 bytes |
| Gosub | 6 bytes |
| Parens Plus Value | 16 bytes |

- 4 -

Level II Error Codes & Messages

| Error Code | Description |
| --- | --- |
| 0 | Next Without For Error |
| 2 | Syntax Error |
| 4 | Return Without Gosub Error |
| 6 | Out of Data Error |
| 8 | Function Code Error |
| 10 | Overflow Error |
| 12 | Out of Memory Error |
| 14 | Undef'd Statement Error |
| 16 | Bad Subscript Error |
| 18 | Redim'd Array Error |
| 20 | Division by Zero Error |
| 22 | Illegal Direct Error |
| 24 | Type Mismatch Error |
| 26 | Out of Space Error |
| 28 | String too Long Error |
| 30 | Formula too Complex Error |
| 32 | Can't Cont Error |
| 34 | No Resume Error |
| 36 | Resume Without Error |
| 38 | Unprintable Error |
| 40 | Missing Operand Error |
| 42 | Bad File Data Error |
| 44 | Disk Command?Syntax Error |

Standard VZ Internal Codes

| Dec | Hex | BASIC Keyword | Exits to BASIC? |
| --- | --- | --- | --- |
| 128 | 80 | End | Jumps to BASIC |
| 129 | 81 | For | No |
| 130 | 82 | Reset | No |
| 131 | 83 | Set | No |
| 132 | 84 | Cls | No |
| 135 | 87 | Next | No |
| 136 | 88 | Data | No |
| 137 | 89 | Input | Yes* |
| 138 | 8A | Dim | No |
| 139 | 8B | Read | No |
| 140 | 8C | Let | No |
| 141 | 8D | Goto | No |
| 142 | 8E | Run | No |
| 143 | 8F | If | No |
| 144 | 90 | Restore | No |
| 145 | 91 | Gosub | No |
| 146 | 92 | Return | No |
| 147 | 93 | REM | No |
| 148 | 94 | Stop | Jumps to BASIC |
| 149 | 95 | Else | No |
| 150 | 96 | Copy (VZ Extension) | Yes |
| 151 | 97 | Color(VZ Extension) | No |
| 152 | 98 | Verify (VZ Extension) | Yes |
| 156 | 9C | Crun (VZ Extension) | Yes |

- 5 -

| 157 | 9D | Mode (VZ Extension) | No |
| --- | --- | --- | --- |
| 158 | 9E | Sound (VZ Extension) | Yes |
| 160 | A0 | Out | No |
| 175 | AF | Lprint | No but corrupts output |
| 177 | B1 | Poke | No |
| 178 | B2 | Print | No |
| 179 | B3 | Cont | Immediate Mode Only |
| 180 | B4 | List | Jumps to BASIC |
| 181 | B5 | Llist | Jumps to BASIC |
| 184 | B8 | Clear | No |
| 185 | B9 | Cload (VZ Extension) | Yes |
| 186 | BA | Csave (VZ Extension) | Yes |
| 187 | BB | New | Clears Program Area |
| 188 | BC | Tab | No |
| 189 | BD | To | No |
| 191 | BF | Using | No |
| 193 | C1 | Usr | No |
| 198 | C6 | Point | No |
| 201 | C9 | Inkey$ | Yes* |
| 202 | CA | Then | No |
| 203 | CB | Not | No |
| 204 | CC | Step | No |
| 205 | CD | + | N/A |
| 206 | CE | - | N/A |
| 207 | CF | * | N/A |
| 208 | D0 | / | N/A |
| 209 | D1 | ^ | N/A |
| 210 | D2 | And | No |
| 211 | D3 | Or | No |
| 212 | D4 | > | N/A |
| 213 | D5 | = | N/A |
| 214 | D6 | < | N/A |
| 215 | D7 | Sgn | No |
| 216 | D8 | Int | No |
| 217 | D9 | Abs | No |
| 219 | DB | Inp | No |
| 221 | DD | Sqr | No |
| 222 | DE | Rnd | No |
| 223 | DF | Log | No |
| 224 | E0 | Exp | No |
| 225 | E1 | Cos | No |
| 226 | E2 | Sin | No |
| 227 | E3 | Tan | No |
| 228 | E4 | Atn | No |
| 229 | E5 | Peek | No |
| 243 | F3 | Len | No |
| 244 | F4 | Str$ | No |
| 245 | F5 | Val | No |
| 246 | F6 | Asc | No |
| 247 | F7 | Chr$ | No |
| 248 | F8 | Left$ | No |
| 249 | F9 | Right$ | No |
| 250 | FA | Mid$ | No |

- 6 -